ESWATINI GOVERNMENT

NATIONAL TRADE STANDARD

APPROVED BY THE INDUSTRIAL TRAINING BOARD

FOR TADE TESTING IN THE TRADE/OCCUPATION

OF WELDER GAS/ELECTRIC.

- Practical Skill Requirement - Grade 3

Every applicant must be able to:

- Observe all safety precautions – personal and equipment.

- Read a measuring rule.

- Use and maintain in good condition the tools and equipment in common use in the tread.

- Recognise all common metals and alloys used in engineering including mild steel, brass, phosphor bronze, copper, cast aluminium, zinc alloy, die-cast alloy and malleable iron.

- Select the correct rods and fluxes for the gas and electric arc.

- Select the correct electrodes for the arc welding of mild steel.

- Prepare work pieces for welding by gas and electric arc.

- Carry out simple brazing in malleable iron, cast iron, and mild steel including the silver-soldering of a brass or steel nipple to a copper pipe.

- Clean and prepare a fuel tank for welding or brazing in such a manner as to eliminate the possibility of an explosion.

- Fabricate a simple welded structure in mild steel working from a simple sketch or pattern using either gas or electric arc.

- Cut simple shapes from mild steel using an Oxy-acetylene torch or an oxy-LPG cutter.

- Join carbon steel using Metal Inert Gas (MIG) welding method.

b) Tools and Equipment Requirements Grade 3

Every applicant should in his possession and/or be able to use tools and equipment supplied as follows:

Engineer’s try square		Measuring rule or tape		Calipers

Files				Hacksaw				Hammers

Wire brush			Hand drill				Screwdrivers

Pliers  				Scriber				Electric Grinder

Tin snips				Electric drill				Welding apron

Welding goggles			Welding gloves		Welding mask/shield

c) Related Knowledge Requirements – Grade III

Every applicant must have a knowledge of:

- Safety precautions – personal and equipment.

- Good house keeping – care and maintenance of tools and equipment.

- The uses of mild steel, cast iron, cast steel, brass, phosphobronze, copper, cast iron, cast aluminium, zinc alloy, die cat alloy and malleable iron

- Welding rods and fluxes used for welding of mild steel by oxy-acetylene and the proper care and storage of welding rods.

- Colour marked electrodes and the use of manufacturers selection charts.

- Brazing and silver-soldering methods and techniques.

- The logical sequence of operations to be followed when fabricating a simple welded structure in mild steel including the preparations of joints prior to welding by either oxy-acetylene or electric arc.

- The methods of cutting simple shapes from mild steel by oxy-acetylene including the cleaning and adge finishing of cuts

Follow us on Facebook

ESWATINI GOVERNMENT

NATIONAL TRADE STANDARD

APPROVD BY THE INDUSTRIAL TRAINING BOARD

FOR TADE TESTING IN THE TRADE/OCCUPATION

OF WELDER GAS/ELECTRIC.

- Practical Skill Requirements Grade 2

In addition to fulfilling the Grade 3 requirements:

Every applicant must be able to:

- Work from a dimensioned drawing and correctly interpret welding symbols.

- Use jigs, clamps and fixtures necessary in welding operations.

- Carry out repairs to machines parts using correct welding build-up techniques in cast iron and steel castings.

- Carry out downhand, vertical and overhead welds in electric-arc and oxy-acetylene.

- Weld low-pressure pipes and other cylindrical objects on site ‘in-site’ by either oxy-acetylene or electric-arc.

- Repair cracks in cast aluminium by either welding process.

- Remove a defective weld using either an oxy-acetylene gouging nozzle or a gouging electrode.

- Cut a variety of shapes from mild steel using oxy-acetylene.

- Carry out various welds avoiding cracks and distortions.

- Operate A.C. and D.C. welding set.

- Carry out all types of brazing and use spot welding machine efficiently.

- Build-up a worn machine part using either steel or hard-facing material and bring the part to approximately the original.

- Join carbon steel using Metal Inert Gas (MIG) welding method.

- Tools and Equipment Requirements - Grade 2

Every applicant should have in his possession and/or be able to use tools and equipment supplied as for Grade 3 plus any other tools equipment required to carry out the practical skill recruitment satisfactorily.

- Related Knowledge Requirements - Grade 2

In addition to fulfilling the Grade 3 requirements;

Every applicant must have sound knowledge of:

- Engineering drawings and welding symbols.

- The fixtures, jigs and clamps used in various welding operations and either applications.

- The procedures and logical sequence of operations to be followed whencarrying out repairs using the build-up process in cast iron and steel castings.

- Pre-heating and slow cooling when welding certain metals.

- The cause of arc blow and hot to counteract it.

- A.C. and D.C. welding sets, either opertation and application to various tasks.

- Gauge sizes of electrodes and the relationship between gauge sizes, amperage and size of workpiece.

- The methods to be followed to avoid cracks and distortions, stress-relieving, etc.

- High and low pressure welding equipment and its application to various tasks.

- Profile cutting techniques.

- Malleability, tenacity, ductility, etc.

Follow Us on Facebook

ESWATINI GOVERNMENT

NATIONAL TRADE STANDARD

APPROVD BY THE INDUSTRIAL TRAINING BOARD

FOR TADE TESTING IN THE TRADE/OCCUPATION

OF WELDER GAS/ELECTRIC.

- Practical Skill Requirements - Grade 1

In addition to fulfilling the Grade 2 and Grade 3 requirements:

Every applicant must be able to:

- Weld all sizes and types of structural steel in all positions using electric-arc, and produce sound welds free from porosity and cracks with a minimum of distortion, good appearance and finish without underout.

- Fabricate a steel structure working from an engineering drawing.

- Cold weld cast iron using the electric-arc, the finished work to be free from cracks and porosity and to be machineable.

- Carry out the gas welding cast iron and aluminium including the repair of cracks, building up missing sections and filling, drilling and threading holes.

- Joint aluminium and mild steel sheet with minimum distortion using oxyacetylene.

- Braze or bronze weld any materials where these processes are normally applicable.

- Joint dissimilar metals such as copper, brass, mild steel, using low temperature brazing alloys and appropriate fluxes.

- Use hard-surfacing materials and build up worn parts to their original shapes.

- Prepare a specimen for a specified test on part welding by the electric-ac method.

- Join carbon steel using Metal Inert Gas (MIG) welding method.

- Join stainless steel and aluminium using Tungsten Inert Gas (TIG) welding method.

- Tools and Equipment Requirement Grade 1

Every applicant should have in his possession/or be able to use tools and equipment supplied as for Grade 2 and 3 plus any other tools and equipment required to carry out the practical skill requirement satisfactorily.

- Related Knowledge Requirements Grade 1

In addition to fulfilling the Grade 2 and 3 requirements:

Every applicant must have a sound knowledge of:

- The application of welding techniques in all position.

- And be able to interpret drawings with welding symbols and plan welding sequences.

- The cold welding process.

- Build-up and repair techniques in relation to cast iron and aluminium.

- Low temperature brazing alloys and fluxes and their application.

- Hard-surfacing materials and their application to various tasks.

- Welding techniques in relation to stainless steel, zinc base alloy, bronzes, fitting tungsten-carbide to machine tools, re-metalling of bearings and metal spraying.

- The operation of a profile cutter or specialised cutting and gouging equipment.

Follow Us on Facebook